RC PLANE DESIGN

PERSONAL USE, I WILL MAKE ANOTHER README DOCUMENT.

BELOW I WILL INDICATE ALL THE CHOICES I HAVE MADE FOR THE DESIGN OF THE PLANE FROM THE MOST IMPORTANT TO THE LEAST IMPORTANT.                         AIRFOIL: the airfoil will be a naca, since the rc plane will not have landing gear I will have to make it take off by pushing it with my hand. for this reason I need an airfoil that has a lot of lift even at low speeds, but it will probably have a lot of drag (since the trailing edge will be flap-shaped) and it will waste a lot of battery

As an airfoil I will use a CLARK Y, very famous in the world of rc planes for its simplicity. the wings will probably be made of petg for its resistance to the sun (useful when you are in the mountains in summer)

ELECTRICAL PARTS: I will use a Brushless motor with a 1045 propeller with an esc with integrated 5v bec, the battery will be a 3s 1300 mAH, flysky receiver and radio.

MATERIALS: I will use petg for its impact and sun resistance with 1% gyroid infill

PROGRAMS I WILL USE: to design the plane I will use autodesk fusion, while for the cfd simulation I will use autodesk cfd